ZF-007使用说明

ZF-007是智峰工作室开发的USB脱机编程下载器。采用精美金属外壳，体积小价格便宜，是目前已知的USB编程下载器中最精致、稳定的。

特点：

- 可通过progisp1.68对ZF-007进行在线升级。

- 可以编写所有atmel的低压isp编程接口的avr单片机，at89s51,at89s52

- 可以通过progisp1.68自定义支持最新型号的芯片的编程。

- 适应电压宽（2.7v-5.5v）

- 可以脱离电脑编写芯片（最大64K/增强型128K）

- 可为目标板提供时钟（对于AVR锁定芯片特别有用）

- 可超低速下载（使用32768HZ的精振）

- 下载速度快（8K程序大约2秒）

- 可配合progisp1.68实现编程项目的工程化管理

- 可以在脱机模式下限制写入次数

编程接口

编程主界面一

- 正常编程的步骤：执行progisp1.68调入编程数据，选择熔丝位,连接好目标板，加电，等待下载编程器绿灯亮,，选择相应的按钮进行操作。（操作时红灯，绿灯交替闪烁，闪烁的速度越快表示下载速度越快）

- 脱机编程的步骤：执行progisp1.68调入编程数据，选择熔丝位，勾选[脱机数据]这时[自动按钮]变为[写入映像]，点击[写入映像]，将编程数据写入下载器。这时就可连接目标板，压下下载器的按钮，如果与目标板连接正确，红灯亮数据正在写入，红灯灭绿灯亮编程完毕。如果出错，红灯闪烁，闪烁的次数代表不同的出错地方。（闪烁3下进入编程模式错误。4是擦除，5是查空。6是写入，8是校验。）

适用范围：

Avr及89Sx单片机的开发编程，生产，现场升级等

问与答：

问：ZF-007可完成熔丝编程及加密功能吗？

答：可以，具体需要编程的选项用POWERPRG来指定

问：ZF-007的售后服务如何？

答：ZF-007免费维修3个月，3个月后维修仅收取元器件费

关于progisp1.68的使用说明见网站。

如有疑问请发邮件到MCU598@163.COM

智峰工作室（www.zhifengsoft.com）

附支持芯片的部分列表

51系列单片机：

AT89S51,AT89S52AT89LS51,AT89LS52,AT89LS53

AVR系列单片机：

AT90S1200,AT90S2313,AT90S2323,AT90LS2323,AT90S2343,AT90LS2343,AT90S4433,AT90LS4433,AT90S8515,AT90S8535,AT90LS8535,ATmega48,ATmega48V,ATmega88,ATmega88V,ATmega168,ATmega168V,ATmega8,ATmega8L,ATmega16,ATmega16L,ATmega32,ATmega32L,ATmega64,ATmega64L, ATmega161,ATmega161L,ATmega163,ATmega163L,ATmega323,ATmega323L,ATmega8515,ATmega8515L,ATmega8535,ATmega8535L,ATmega103,ATmega103L,ATmega128,ATmega128L,ATmega162,ATmega162L,ATmega169,ATmega169L, ATtiny12,ATtiny12L,ATtiny13,ATtiny13LATtiny15,ATtiny15L,ATtiny22,ATtiny22L,ATtiny26,ATtiny26L